GNSS COORDINATE TIME-SERIES ANALYSIS FOR TECTONIC MOVEMENT DETECTION – THE CASE OF ACRG00GHA

ABSTRACT

1 INTRODUCTION

Tectonic movement such as plate shift, faults slips and crustal deformations, plays play a critical role in monitoring the earth surface and studying seismic activities. Slow slip event, the most commonly observed form of transient, has been documented in many active faults system throughout studies across the world (jolivert, 2020). Study has shown that a slow slip on faults can serve as potential warning signal for impending large-scale earthquakes (Kato at al., 2012; Bouchon at all., 2013; Socquet at al., 2017). Determination of the characteristics of transient tectonic movement mainly relies on ground deformation data obtained from daily time-series displacement monitoring from Global Navigation Satellites System (GNSS) (Mastella et al., 2025; Bevis at al., 2014). Daily time series is more applicable for capturing larger temporal variations in plate motion (materna et al., 2019; Yuzariyadi at al., 2021). Understanding of these movement is necessary for geohazard assessment. The introduction of global navigation satellites systems has revolutionized the way researchers and scientist monitor and analyze tectonic activities.

For the past century, the major methods for deriving datasets used for such a purpose were traditional geological field surveys, land surveying techniques and seismic motion sensors (Thatcher, 2003; Savage et al., 1973). With the development of space-based sensor, the Global positioning system (GPS) and later the broader categories of Global Navigation Satellites Systems (GNSS), marked a paradigm shift in crustal movement determination (Thatcher, 2003).

Ghana lies on the west Africa craton, a geologically stable block of the African plate (Osei et al., 2018, Boateng at al., 2020). Despite these stabilities localized crustal movement occur due to regional stress, seismic activities along the Atlantic trans for fault and geodynamic process (Osei et al., 2018). This subtle deformation is difficult to detect without high precision GNSS monitoring network (Amponsah et al., 2012; Nartey et al., 2021). Monitoring these movement is vital for infrastructure safety, resource management and advancing geoscience studies in west Africa.

GNSS provides high-precision, continuous positioning data from ground-based receivers, enabling researchers to track subtle displacements in the Earth's crust over time (Larson, 2009). The rapid development of space geodesy, monitoring of tectonic activities and deformation are now easily monitored using GNSS data (Quarcoo et al., 2024). Continuous GNSS provide data with precise and high Spatio-temporal resolution (Wonnacott et al., 2011). The purpose of this study is to;

- Quantify crustal deformation rates;

- Estimation of velocity vectors (horizontal and vertical velocities); and

- Identify and model transient deformation.

2. Data and Study Area

2.1 Data Source and description

Daily GNSS observation and navigation data for the station ACRG00GHA were downloaded from the international GNSS service (IGS) data archive, with the major repository being the IGS data center.

The downloaded data comprises of

1. Receiver Independence Exchange format (RINEX) observation files at 30 seconds sampling rate. These files contain code and carrier phase measurement from multiple GNSS constellation.

2. Broadcast ephemeris navigation files in RINEX format, corresponding to each day of observation.

Data for the entire year of 2024 were acquired, covering from January 1, 2024 (DOY 001) to 31 December, 2024 (DOY 336). This covers a full year dataset for detecting seasonal variations.

The station ACRG00GHA (Accra, Ghana) was selected as it is situated within the west African and the Ghanaian coastal plains.

The station metadata, as per the IGS website is summarized below

Latitude                                                  5.641472

Longitude                                              -0.207111

Height                                                     83.4m

Constellation RINEX                             GPS +GLO+ GAL+ BDS+ IRNSS+ SBAS

Constellation - real time                         GPS +GLO+ GAL

Clock                                                      Internal

- Literature Review

- Methods Used

  - Data preprocessing.

Each RINEX data were first submitted to the Canadian Spatial Reference System – Precise Point Positioning, (CSRS-PPP) online service in static mode. CSRS-PPP handles cycle slip detection, ambiguity resolution and coordinate in International Terrestrial Reference Frame (ITRF).  The position (.pos) file, provides daily Cartesian (X, Y, Z) and geodetic (latitude, longitude, height) coordinates.

  - Time Series Cleaning and Outlier Removal

The daily coordinate time series were loaded to MATLAB. Outlier detection was accessed using a 3-sigma (3σ) criterion. The application was done iteratively for each component and the median and median absolute deviations were calculated. All epoch with residual exceeding 3σ were removed.

## 4.3 Step Detection and Correction

Non-tectonic jumps as a results of antenna changes, cable replacements or firmware updates were calculated using the sequential t-test analysis of regime shifts (STARS) algorithm in MATLAB. A 30 days window and a significance level of α = 0.05 was used to detect the offset. Each offset was crosschecked with station logs metadata and earthquake catalogs from United State Geological Survey (USGS) to confirm its non-tectonic origin. the magnitude of each confirmed offset was then estimated within a 60-day window. Removal of offset was done by subtracting the step amplitude from all subsequent epochs, creating a step corrected time series.

## 4.4   Noise Characterization and Spectral Analysis

A preliminary least-square fit was applied to remove seasonal signals and to determine noise (white, flicker and random). Residual analysis was conducted using the Welch average periodogram (pwelch) with a humming window, 50% overlap and 2048 FFT points. Log-power spectral density (PSD) was plotted against log-frequency and slope of the log-PSD calculated using a linear regression of frequency range 0.5 to 50 cycles.  A zero indicates white noise, near – 1 indicates flicker noise and near – 2 indicates random walk. Maximum Likelihood Estimation (MLE) was used to jointly estimate the amplitude of all noises to best fit the observed residual covariance structure. The MLE provides the parameters to build a realistic noise covariance matrix for final velocity estimation.

## 4.5 Deterministic Signal Modeling

The deterministic model for each coordinate component was expressed as:

fx=a0+k=12cksin(2πkt)+dkcos2πkt+j   sjH(t-TJ)    + Ɛ(t)

where:

a = intercept,

b = linear velocity (tectonic signal, mm/yr.),

k=1 (annual) and k=2 (semi‑annual) seasonal terms,

sj= amplitude of the j-th offset at time T_j (from Step 3),

H = Heaviside step function,

Ɛ(t)= colored noise (white + flicker).

## 4.6 Velocity Estimation with Realistic Uncertainties and Validation

The final velocities and its uncertainties were estimated using weighted least squares based           on the noise covariance matrix derived from the MLE.

Two validation procedure was used to assess the reliability of the estimated tectonic velocities;

- Bootstrap residual resampling. The final model was randomly resampled and added back to the model. The entire velocity estimation was repeated 1000 times. The 2.5th and 97.5th percentiles of the resulting velocity distribution defined the 95% confidence interval. The narrow interval compared to the formal uncertainties indicated robust estimation.

- Synthetic time-series test. A single synthetic time-series was generated with a known input velocity, realistic offset and colored noise parameter matching those estimated from the real data. The complete processing chain was applied to the generated series. The recovered velocity was compared to the input values. A smaller difference confirms that the retrieved signal are accurately determined